Architectural Consideration of Join_unit

Abstract

1. Introduction

*T is a multithread parallel computer using Motorolla MC88110. The

basic idea of *T is to add ability to switch quickly between threads

onto conventional RISC type microprcessor which has great single

thread performance [1, 2].

There will be wide variations to realize *T node processor based on

this idea: from adding an autonomous processor, called synchronization

processor [1, 2], to adding minimal hardware support for message

handling. Obviously, it is a tradeoff between hardware cost (and

possively design cost) and performance. The study described in this

report positons in the middle of this wide spectrum: utilizing special

function unit (SFU) allowed in MC88000 family architecture to realize

the ability explained above.

Specifically, this report focuses on basic idea on how to speed up

join operation with hardware support. We call this hardware join_unit.

As join operation is exactly a synchronization between threads (please

refer to Fig. 1), it is quite important to realize fast join

operation.

In the next section, loose definition of SFU and basic idea of

join_unit before this work is presented. Alternatives for the

join_unit are given in Section 3, and a little detailed implemantation

of the best promising alternative is shown in Section 4.

Please note that the work described in this report is preliminary.

Complete version of the join_unit (possively along with other hardware

primitives) will be reported elsewhere.

2. Background

2.1. SFU

Fig. 2 shows a block diagram of MC88100 and MC88200. Readers can think

of MC88110 as integrating these blocks on one chip and change it into

superscalar machine, i.e., double instruction issue machine.

Special function unit (SFU) is indicated by dash-dotted block in this

figure. It has 2 inputs (64b each) from source 1,2 buses and one

output to destination bus (note that actual SFU in MC88110 has 2 pairs

of this i/o ports because of double instruction issueing). Obvious but

important thing to note here is the connection of each blocks to

internal buses. For example, data from data unit cannot be directly

given to SFU: It should be first stored into register file.

As far as obeying this constraints, modification caused by SFU might

only be a indtruction decoder. Thus the implementation would be quite

easy.  In the next section, several alternatives are given, some of

them viorates the constraints.

2.2. Join Operation without Hardware Support

As a base case, let us consider the case when there is no hardware

support for join operation exept a message_buffer, which stores

messages from network.

Possible code sequence for this case is shown in Fig. 3.

So, our goal is to reduce this code as much as possible by moderate

hardware support.

2.2. Original Idea for Join_unit

Fig. 4 shows the original idea for join_unit before this study. The

key idea here is to implement join_cache to store join counters.

Though this Idea seems to be fine, some problems can be pointed out.

The first problem is a hardware cost.

Thus

3. Alternatives

3.0. No Hardware support

As a base case, let's consider the case when there is no hardware

support.

Our goal is to reduce this cycle number as

3.1. Join_cache elimination

3.2. Adding direct path from Data_unit to Join_unit

3.3. Message Pipelining

3.4. Sequencer Modification

3.4. Summary of Alternatives

4. Hardware Consideration

5. Summary

4.